Eye-Gaze Communication App

QUICK START GUIDE  ·  5-Minute Essentials

Once the app is installed on a smartphone or tablet, the user launches it and is shown a disclaimer and a short set of instructions. After accepting the disclaimer, the user begins a session.

The camera preview should show the eye region of the face clearly — this confirms the app is tracking correctly. From there, the user selects a phrase from the phrase board using eye movements alone.

Every phrase on the board is mapped to a binary codeword. The user builds that codeword one digit at a time: a glance LEFT enters 0, a glance RIGHT enters 1. Holding the gaze at the CENTER for two seconds ends the entry, and the device speaks the selected phrase aloud.

No special hardware. No keyboard. No spelling. Just a Galaxy phone or tablet and its front camera.

## The three gaze movements

| Look LEFT | = 0 | Adds the digit 0 to the code |
| --- | --- | --- |
| Look RIGHT | = 1 | Adds the digit 1 to the code |
| Look CENTER | 2 sec | Confirms — the phrase is spoken aloud |

Example: "Uncomfortable" = 011 → LEFT, RIGHT, RIGHT, then CENTER for 2 seconds.

| LEFT (0) | RIGHT (1) | RIGHT (1) | CENTER (2 s) |
| --- | --- | --- | --- |

Made a mistake? Just look at the center for 2 seconds. Codes that do not exist are thrown away automatically, and you can start again.

Safety: This is a communication aid, not a medical device. A caregiver must always be present, and there must always be a backup way to call for help.

# 1.  Setting Up

| Mounting position | Position the device - About 30 cm from the user's face - Camera at eye level, slightly above the line of sight so it does not block the view - Face centred in the camera preview - Light in FRONT of the user — never a window behind them - Wipe the camera lens before use |
| --- | --- |

Glasses: remove them if possible. If they must be worn, tilt the device until no glare shows on the lenses in the camera preview — reflections block eye detection.

## First launch — 3 taps

| 1 | Choose the language at the top of the screen: English or 한국어. This sets the menus, the phrase list and the voice. |
| --- | --- |
| 2 | Read the disclaimer and tap Accept. |
| 3 | On the Instructions screen choose the device type — Galaxy Phone or Galaxy Tab — so the app knows where the camera is. |

## The three buttons

| Button | What it does |
| --- | --- |
| Start | Turns the camera on and starts tracking. The status light turns GREEN (TRACKING). |
| Pause / Resume | Suspends tracking; the camera stays on. Status light turns ORANGE (PAUSED). |
| Stop | Turns the camera off and stops everything. |

The user can pause without help. Closing the eyes for five seconds or longer pauses tracking; closing them again for five seconds resumes it. Use this while resting or sleeping, or whenever the user is not composing a phrase so that ordinary eye movements are not read as input

## Which orientation?

- Galaxy Tab: use LANDSCAPE. In portrait the camera sits far to the side and accuracy drops.

- Galaxy Phone: either orientation works.

Before the user starts: the caregiver should try the app first. Ten minutes of practice makes teaching much easier.

# 2.  The Main Phrase Chart

This is the only page most people need. Short codes are the most urgent phrases — emergencies take a single look.

| Code | Gaze | Says |
| --- | --- | --- |
| 0 | L → centre | Can't breathe |
| 1 | R → centre | Suction |
| 00 | L L → centre | Yes |
| 01 | L R → centre | No |
| 10 | R L → centre | Repeat |
| 11 | R R → centre | Wait |
| 000 | L L L → centre | Thirsty ★ |
| 001 | L L R → centre | Urinate ★ |
| 010 | L R L → centre | Defecate ★ |
| 011 | L R R → centre | Uncomfortable ★ |
| 100 | R L L → centre | opens the PAIN page |
| 101 | R L R → centre | opens the ACTION page |
| 110 | R R L → centre | opens the ENVIRON · STATE page |
| 111 | R R R → centre | opens the FEELING page |

| Red | Emergency — one look only |
| --- | --- |
| Green | Yes / No / Repeat / Wait — answering questions |
| Yellow | ★ Shortcuts — these four slots change by themselves (see below) |
| Blue | Doorways to the four category pages |

## ★ The yellow shortcuts learn from use

- Every time a phrase is chosen from a category page, its counter goes up by one.

- The four most-used phrases automatically move into the yellow slots (000–011), so frequent needs get shorter codes.

- Until enough phrases have been used, the defaults stay: Thirsty, Urinate, Defecate, Uncomfortable.

- To see or clear the counters: Setup → Zone 3A – Usage Frequency → Show / Reset all usage counts.

Teaching order: practise the red and green phrases first (1–2 looks) until they are reliable. Add the yellow shortcuts next, then introduce the category pages one at a time. Sessions of 10–15 minutes work better than long ones.

# 3.  The Four Category Pages

Entering a 3-digit code opens a category page. The counter then resets to empty, and the user enters a second, shorter code to select a phrase within that category. After the phrase is spoken the app returns to Main by itself. Looking LEFT + centre (code 0) always goes back to Main.

| PAIN enter code 100 (R L L). 0; ← back to Main. 1; Stomachache. 00; Headache. 01; Back pain. 10; Neck pain. 11; Chest pain. 000; Nausea. 001; Mouth dry. 010; Eyes dry. 011 – 111; empty — free to add. | ACTION enter code 101 (R L R). 0; ← back to Main. 1; Medicine. 00; Change posture. 01; Turn me. 10; Lift head. 11; Lower head. 000; Wipe. 001; Cover me. 010; Call nurse. 011; Come here. 100; Sit. 101; Stay still. 110; Rest. 111; Urinate. |
| --- | --- |
| ENVIRON · STATE enter code 110 (R R L). 0; ← back to Main. 1; Cold. 00; Hot. 01; Too bright. 10; Too dark. 11; Too loud. 000; Sleepy. 001; Itchiness. 010; Thirsty. 011; Uncomfortable. 100 – 111; empty — free to add. | FEELING enter code 111 (R R R). 0; ← back to Main. 1; Thank you. 00; Sorry. 01; Anxious. 10; Scared. 11; Sad. 000; Lonely. 001; Happy. 010; Exhausted. 011; Angry. 100; Miss you. 101 – 111; empty — free to add. |

Adding your own phrases: Instructions screen → Setup → tap any empty *** slot → type the phrase → Save, then Save & Return. Keep phrases to 1–3 words so the voice stays clear. Three items cannot be changed: "Can't breathe" (code 0), "back to Main" (code 0 on category pages), and the four category doorways (100–111).

# 4.  Four Worked Examples

| To say | Do this |
| --- | --- |
| "Can't breathe" | LEFT → centre 2 s. Spoken immediately. |
| "Headache" | RIGHT LEFT LEFT → centre 2 s (PAIN page opens) → LEFT LEFT → centre 2 s. |
| "Thank you" | RIGHT RIGHT RIGHT → centre 2 s (FEELING page opens) → RIGHT → centre 2 s. |
| Go back to Main | LEFT → centre 2 s. Nothing is spoken. |

# 5.  If Something Is Not Working

| Problem | What to do |
| --- | --- |
| Camera will not start | Android Settings → Apps → ALS Eye-Gaze → Permissions → allow Camera. |
| No voice | Check the volume buttons. Then Android Settings → General Management → Language & Input → Text-to-Speech. |
| Gaze not detected | More light. Clean the lens. Move the device closer. Remove glasses, or tilt the device until the glare disappears. |
| Reacts to every small movement | Check the eyelids are fully open, not half-closed. Ordinary small eye movements should not enter a digit. |
| Wrong phrase was spoken | Use Repeat (10) or Wait (11) to signal the caregiver, then enter the intended code again. |
| Unexpected phrases in the yellow slots | Setup → Zone 3A – Usage Frequency → Reset all usage counts. |
| Phrases in the wrong language | Go back to the Disclaimer screen and select the language again. |

Safety notice This application is a communication aid only. It is not a medical device and must never be the only way the user can alert someone in an emergency. - Use only with a caregiver present and monitoring. - Always keep a backup alert method within reach. - Have a speech-language pathologist assess the user before long-term use. The developers accept no liability for consequences arising from use of this application. By using it you accept these risks.

This is a short guide. The full User Manual contains the complete Setup screen reference, screen layouts for every device and orientation, and detailed training guidance.

Questions, problems, or feedback: contact.eyeum@gmail.com